TNPSC CTS DIPLOMA LEVEL – TEXTILE TECHNOLOGY

Spun Yarn Formation — 100 MCQ Question Bank

SECTION A — EASY (1–25)

Question 1

What is the main objective of ginning?

A) To twist yarn                                                C) To produce roving

B) To separate fibre from seeds                        D) To form fabric

Correct Answer: B) To separate fibre from seeds

Explanation: Ginning separates cotton fibre from seeds without damaging the fibre.

Question 2

Ginning is defined as the process of:

A) Combining fibres and seeds

B) Separating fibre (lint) from seeds without damage to the fibre

C) Adding twist to fibre

D) Removing trash only

Correct Answer: B) Separating fibre (lint) from seeds without damage to the fibre

Explanation: This is the exact definition of ginning given in the PDF.

Question 3

In Roller Ginning, fibres are separated from seeds through which action?

A) Suction

B) Push-pull-hit (stretching) action

C) Compression

D) Chemical treatment

Correct Answer: B) Push-pull-hit (stretching) action

Explanation: An oscillating/reciprocating/rotary knife beats the seeds, separating fibres by a stretching (push-pull-hit) action.

Question 4

In Saw Ginning, fibres are pulled away from seeds by:

A) A stationary knife

B) A leather roller

C) Rotating circular saws

D) A spiked cylinder alone

Correct Answer: C) Rotating circular saws

Explanation: Rotating circular saws that project between ribs pull the fibres from seeds at high speed.

Question 5

The main types of ginning machines include all EXCEPT:

A) Knife Roller Gin

B) Saw Gin

C) McCarthy Gin

D) Rotor Gin

Correct Answer: D) Rotor Gin

Explanation: The PDF lists only Knife Roller Gin, Saw Gin, and McCarthy Gin as ginning machine types.

Question 6

The McCarthy Gin uses which type of roller?

A) Steel roller

B) Leather or composition roller

C) Wooden roller

D) Ceramic roller

Correct Answer: B) Leather or composition roller

Explanation: McCarthy/Single Roller Gin utilises a leather or composition roller touching a stationary knife.

Question 7

What is the approximate production rate of Roller Gins?

A) 100 kg/hour

B) 250 kg/hour

C) 1000 kg/hour

D) 2000 kg/hour

Correct Answer: B) 250 kg/hour

Explanation: Roller Gins produce around 250 kg/hour (600–750 lbs of lint per hour).

Question 8

Saw Gins can produce up to how much cotton per hour?

A) 250 kg

B) 500 kg

C) 2000 kg

D) 5000 kg

Correct Answer: C) 2000 kg

Explanation: Saw Gins produce 500 to 2000 kg per hour (13–15 bales of lint per hour).

Question 9

Opening is defined as the process of:

A) Removing impurities

B) Breaking compressed, matted fibre mass/bales into smaller tufts

C) Combining fibres

D) Inserting twist

Correct Answer: B) Breaking compressed, matted fibre mass/bales into smaller tufts

Explanation: This matches the PDF's definition of opening.

Question 10

Cleaning in the blowroom removes:

A) Twist

B) Impurities such as trash, dirt, seeds, leaf particles, dust

C) Moisture only

D) Colour

Correct Answer: B) Impurities such as trash, dirt, seeds, leaf particles, dust

Explanation: Cleaning removes impurities from the opened cotton fibres.

Question 11

Mixing is defined as combining:

A) Only similar fibres with exactly known properties

B) Several bales of similar materials to achieve a generally known average property

C) Yarns after spinning

D) Fabrics after weaving

Correct Answer: B) Several bales of similar materials to achieve a generally known average property

Explanation: This is the PDF's definition of mixing.

Question 12

Blending differs from mixing because it involves:

A) Random combination

B) Precise, measured, and controlled proportions based on measured fibre properties

C) No fibre property measurement

D) Only cotton fibres

Correct Answer: B) Precise, measured, and controlled proportions based on measured fibre properties

Explanation: Blending is combining materials with precise, measured, controlled proportions (e.g., length, fineness, strength).

Question 13

The Blowroom is described as:

A) The final stage of spinning

B) The first section/stage in short-staple mechanical spinning

C) A weaving process

D) A dyeing process

Correct Answer: B) The first section/stage in short-staple mechanical spinning

Explanation: The Blowroom is where compressed bales are progressively opened, cleaned, blended, and prepared for carding.

Question 14

In the Scutcher/Lap Former, the beater rotates at what speed range?

A) 100–300 rpm

B) 750–1100 rpm

C) 5000–8000 rpm

D) 18000–22000 rpm

Correct Answer: B) 750–1100 rpm

Explanation: The Bladed or Kirschner beater rotates at 750–1100 rpm.

Question 15

Chute Feed is also known as:

A) Lap feed	C) Roving feed

B) Tuft feed system	D) Sliver feed

Correct Answer: B) Tuft feed system

Explanation: Chute Feed (Tuft Feed) System transports material pneumatically as loose tufts.

Question 16

Carding is defined as reducing tufts of fibres into:

A) A twisted yarn	C) A woven fabric

B) A filmy web of individual fibres	D) A dyed sliver

Correct Answer: B) A filmy web of individual fibres

Explanation: Carding reduces entangled fibre tufts into a filmy web of individual fibres.

Question 17

Which is NOT an objective of carding as per the PDF?

A) Opening tufts to individual fibres	C) Converting lap into sliver

B) Cleaning impurities	D) Inserting real twist

Correct Answer: D) Inserting real twist

Explanation: Carding objectives include opening, cleaning, converting lap to sliver, and blending — twist insertion is not a carding objective.

Question 18

Drawing refers to the operation of:

A) Twisting only	C) Weaving of slivers

B) Doubling and roller drafting of slivers	D) Bleaching slivers

Correct Answer: B) Doubling and roller drafting of slivers

Explanation: Drawing is defined as the operation involving doubling and roller drafting of slivers.

Question 19

Doubling at the draw frame means combining several slivers and:

A) Reducing count with a draft not equal to slivers combined	C) Adding twist

B) Attenuating with a draft equal in number to the slivers combined, giving one sliver of similar count	                                  D) Removing fibres

Correct Answer: B) Attenuating with a draft equal in number to the slivers combined, giving one sliver of similar count

Explanation: This is the PDF's definition of doubling at the draw frame.

Question 20

Comber Preparatory is performed to convert round carded slivers into:

A) A flat, uniform, condensed sheet (lap)	C) A finished yarn

B) A twisted roving	D) A woven fabric

Correct Answer: A) A flat, uniform, condensed sheet (lap)

Explanation: Comber Preparatory converts round carded slivers into a flat, uniform, condensed lap suitable for the comber.

Question 21

The Comber is used in fine yarn spinning mainly to:

A) Insert twist	C) Blend colours

B) Eliminate short fibres, impurities, and neps	D) Increase yarn weight

Correct Answer: B) Eliminate short fibres, impurities, and neps

Explanation: The Comber eliminates a pre-determined quantity of short fibres (noil), impurities, and neps.

Question 22

The Speed Frame is also known by which other names?

A) Roving Frame, Fly Frame, Simplex	C) Ring Frame, Rotor

B) Draw Frame, Card, Comber	D) TFO, Doubler

Correct Answer: A) Roving Frame, Fly Frame, Simplex

Explanation: The Speed Frame is also called Roving Frame, Fly Frame, or Simplex.

Question 23

Ring spinning converts roving into fine yarn by:

A) Only twisting	C) Only winding

B) Drafting, twisting, and winding	D) Only drafting

Correct Answer: B) Drafting, twisting, and winding

Explanation: Ring spinning is a mechanical system that converts roving into yarn by drafting, twisting, and winding.

Question 24

Doubling (Plying) refers to:

A) Removing twist from single yarns	C) Reducing yarn count

B) Combining two or more single yarns and twisting them together D)Bleaching yarn

Correct Answer: B) Combining two or more single yarns and twisting them together

Explanation: Doubling/Plying combines two or more single yarns which are then twisted together.

Question 25

Rotor spinning is also known as:

A) Ring spinning	C) Air-jet spinning

B) Open-End spinning	D) Vortex spinning

Correct Answer: B) Open-End spinning

Explanation: The PDF titles rotor spinning as "Rotor Spinning (Open-End)".

SECTION B — MEDIUM (26–50)

Question 26

Compared to saw ginning, roller ginning:

A) Produces more short fibre content	C) Is faster

B) Is gentler and preserves maximum fibre length	D) Is not suitable for ELS cotton

Correct Answer: B) Is gentler and preserves maximum fibre length

Explanation: Roller ginning is a gentle process compared to saw ginning and preserves maximum natural fibre length parameters.

Question 27

Roller Gins / McCarthy Gins are preferred for:

A) Short-staple cotton only	C) Only synthetic fibres

B) Handpicked, low-trash cottons and medium, long, and extra-long staple (ELS) varieties	D) Only waste cotton

Correct Answer: B) Handpicked, low-trash cottons and medium, long, and extra-long staple (ELS) varieties

Explanation: The PDF states Roller Gins are preferred for such varieties as Sea Island, Egyptian, American Egyptian, and Pima cottons.

Question 28

Saw Gins are mainly suitable for:

A) Extra-long staple cotton	C) Wool

B) Short-staple and medium-staple cotton varieties	D) Silk

Correct Answer: B) Short-staple and medium-staple cotton varieties

Explanation: Saw Gins are suitable primarily for short-staple and medium-staple cotton varieties.

Question 29

In a Step Cleaner, how many beaters are typically used?

A) 2 to 3	C) 8 to 10

B) 4 to 6	D) 12 to 15

Correct Answer: B) 4 to 6

Explanation: The Step Cleaner is inclined at 45° with 4 to 6 beaters.

Question 30

In the Uni-Mixer, how many points of blending are described?

A) One	C) Three

B) Two	D) Four

Correct Answer: C) Three

Explanation: The PDF describes First, Second, and Third Points of Blending in the Uni-Mixer.

Question 31

The Mono Cylinder Cleaner has a cleaning efficiency of about:

A) 5–10%	C) 50–60%

B) 25–30%	D) 80–90%

Correct Answer: B) 25–30%

Explanation: The Mono Cylinder Cleaner has cleaning efficiency of 25%–30% at 700 rpm.

Question 32

The Multi-Mixer (Trutzschler) contains how many vertical chambers?

A) 2 to 4	C) 10 to 12

B) 6 to 8	D) 16 to 20

Correct Answer: B) 6 to 8

Explanation: The Multi-Mixer contains 6 to 8 vertical chambers with light-barrier height controls.

Question 33

Which is an advantage of the Chute Feed System?

A) Requires more manpower	C) Increases lap faults

B) Eliminates lap formation, doffing, and manual transportation	D) Requires equal blowroom and card running hours only for cleaning

Correct Answer: B) Eliminates lap formation, doffing, and manual transportation

Explanation: Chute Feed eliminates lap formation, doffing, manual transportation, and lap piecing, and reduces manpower requirements.

Question 34

A limitation of the Chute Feed System is:

A) It is difficult to control medium- and long-term sliver weight variations compared to weighing individual laps	C) It requires manual lap piecing

B) It produces more lap faults	D) It cannot be used with pneumatic transport

Correct Answer: A) It is difficult to control medium- and long-term sliver weight variations compared to weighing individual laps

Explanation: This is listed as a limitation of the Chute Feed System in the PDF.

Question 35

In Carding Action, the teeth of the two opposing surfaces:

A) Face the same direction	C) Are stationary

B) Face opposite directions, with surface speed v1 greater than v2 moving in the opposite direction	D) Rotate at equal speed

Correct Answer: B) Face opposite directions, with surface speed v1 greater than v2 moving in the opposite direction

Explanation: Carding action requires opposing tooth direction and v1 > v2 (opposite direction), resulting in fibre separation.

Question 36

The Cylinder to Doffer setting in carding is approximately:

A) 1 mm	C) 5 mm

B) 0.1 mm (0.1–0.125 mm)	D) 10 mm

Correct Answer: B) 0.1 mm (0.1–0.125 mm)

Explanation: The PDF specifies an extremely narrow distance of about 0.1 mm between cylinder and doffer.

Question 37

The purpose of the Heel and Toe Arrangement in carding is to:

A) Increase production speed	C) Reduce fibre feed

B) Ensure gradual carding with a wider setting at the back and closer setting at the front	D) Add twist to sliver

Correct Answer: B) Ensure gradual carding with a wider setting at the back and closer setting at the front

Explanation: This arrangement provides a wider flat-to-cylinder setting where cotton enters and a closer setting where it leaves, for gradual carding.

Question 38

The Licker-in in carding operates at a surface speed of about:

A) 5 m/sec	C) 50 m/sec

B) 21 m/sec	D) 100 m/sec

Correct Answer: B) 21 m/sec

Explanation: The licker-in has a surface speed of 21 m/sec and a draft of 1600.

Question 39

The purpose of the doubling principle at the draw frame is to:

A) Increase twist	C) Increase fibre length

B) Reduce mass variation by averaging thick and thin places across several strands	D) Remove neps only

Correct Answer: B) Reduce mass variation by averaging thick and thin places across several strands

Explanation: Multiple slivers are fed side by side, and by averaging thick/thin places, mass variation is reduced.

Question 40

The draw frame removes what percentage of incoming dust through drafting?

A) Less than 20%	C) More than 80%

B) About 50%	D) 100%

Correct Answer: C) More than 80%

Explanation: The draw frame extracts more than 80% of incoming dust through fiber-on-fiber and fiber-on-metal friction.

Question 41

The "critical drafting region" for cotton at the draw frame lies within a draft of:

A) 1.15 to 1.4	C) 8 to 12

B) 4 to 8	D) 12 to 20

Correct Answer: A) 1.15 to 1.4

Explanation: Operating in this critical drafting region causes stick-slip phenomena and drafting waves.

Question 42

The Hook Reversal Principle in Comber Preparatory requires an even number of passages because:

A) Rectilinear combers straighten and remove only leading hooks	C) Hooks do not affect combing

B) Rectilinear combers remove trailing hooks only	D) Combers remove all hooks regardless of type

Correct Answer: A) Rectilinear combers straighten and remove only leading hooks

Explanation: Since carded slivers have more than 50% trailing hooks and combers remove only leading hooks, an even number of passages (usually 2) reverses hook direction before combing.

Question 43

The Sliver Lap Machine typically doubles how many slivers?

A) 6	C) 60

B) 16–32 (usually 24)	D) 100

Correct Answer: B) 16–32 (usually 24)

Explanation: The Sliver Lap Machine doubles 16–32 slivers, usually 24.

Question 44

The Ribbon Lap Machine typically doubles how many laps?

A) 2	C) 16

B) 6	D) 32

Correct Answer: B) 6

Explanation: The Ribbon Lap Machine doubles 6 laps.

Question 45

In Comber types based on feed direction, Backward/Counter Feed is used for:

A) Short staple fibres with low noil	C) Only synthetic fibres

B) Long staple fibres, producing higher-quality combing with higher noil (14–25%)	D) Waste cotton

Correct Answer: B) Long staple fibres, producing higher-quality combing with higher noil (14–25%)

Explanation: Backward/Counter Feed is used for long staple fibres and produces higher-quality combing with noil of 14–25%.

Question 46

In "Full/Super Combing", the noil percentage is:

A) Up to 5%	C) 10–18%

B) 5–12%	D) Above 18% (up to about 22% or more)

Correct Answer: D) Above 18% (up to about 22% or more)

Explanation: Full/Super Combing removes above 18% noil, up to about 22% or more.

Question 47

In the Speed Frame Bobbin Leading System:

A) Flyer speed is greater than bobbin speed	C) Both speeds are equal

B) Bobbin speed is greater than flyer speed and decreases as bobbin diameter increases	D) Bobbin speed increases as diameter increases

Correct Answer: B) Bobbin speed is greater than flyer speed and decreases as bobbin diameter increases

Explanation: In the Bobbin Leading System, Nbobbin > Nflyer, and bobbin speed gradually decreases as bobbin diameter increases.

Question 48

The draft capability of Ring Spinning can go up to:

A) 12	C) 70–75

B) 22	D) 200

Correct Answer: C) 70–75

Explanation: Ring spinning draft capability is up to 70–75 as per the PDF's Features section.

Question 49

In the Two-for-One (TFO) Twister principle:

A) One twist is inserted per spindle revolution	C) No twist is inserted

B) Two twists are inserted for every one spindle revolution	D) Twist is inserted only during winding

Correct Answer: B) Two twists are inserted for every one spindle revolution

Explanation: This is the fundamental principle of the Two-for-One (TFO) twister.

Question 50

Compared to ring spinning, rotor spinning production is:

A) 6–10 times higher	C) Lower

B) Equal	D) 50% lower

Correct Answer: A) 6–10 times higher

Explanation: Rotor spinning production is 6–10 times higher than ring spinning.

SECTION C — MODERATE (51–75)

Question 51

Using the formula CV_delivered = CV_fed / √N, if CV_fed = 4% and N = 6 doublings are used at the draw frame, the approximate CV_delivered is:

A) 24%	C) 0.67%

B) 1.63%	D) 4%

Correct Answer: B) 1.63%

Explanation: CV_delivered = 4 / √6 = 4 / 2.449 ≈ 1.63%.

Question 52

Using the Total Draft Formula V_total = v_D / v_F, if delivery speed (v_D) = 800 m/min and feed speed (v_F) = 100 m/min, the total draft is:

A) 4	C) 80,000

B) 8	D) 0.125

Correct Answer: B) 8

Explanation: V_total = 800 / 100 = 8.

Question 53

Using the Forward Feed Noil formula N% = [1 − (D − F/2)/L]² × 100, if D = 40 mm, F = 6 mm, L = 40 mm, the noil percentage is approximately:

A) 0.56%	C) 56%

B) 5.6%	D) 92.5%

Correct Answer: A) 0.56%

Explanation: D − F/2 = 40 − 3 = 37; 37/40 = 0.925; 1 − 0.925 = 0.075; 0.075² × 100 ≈ 0.56%.

Question 54

Using the Backward Feed Noil formula N% = [(D + F/2)/L]² × 100, if D = 8 mm, F = 6 mm, L = 40 mm, the noil percentage is approximately:

A) 7.56%	C) 75.6%

B) 27.5%	D) 2.75%

Correct Answer: A) 7.56%

Explanation: D + F/2 = 8 + 3 = 11; 11/40 = 0.275; 0.275² × 100 ≈ 7.56%.

Question 55

The Combing Efficiency formula uses which of the following parameters?

A) Spindle speed and yarn delivery	C) Draft and doubling only

B) 50% span length of comber sliver, 50% span length of comber lap, and noil % extracted	D) CV_fed and N

Correct Answer: B) 50% span length of comber sliver, 50% span length of comber lap, and noil % extracted

Explanation: Combing Efficiency (%) = [(50% span length of comber sliver − 50% span length of comber lap) ÷ (50% span length of comber lap × Noil % extracted)] × 100.

Question 56

Using the TFO formula TPM = (Spindle Speed × 2) / Yarn Delivery, if spindle speed = 12,000 rpm and yarn delivery = 30 m/min, the TPM is:

A) 400	C) 200

B) 800	D) 12,000

Correct Answer: B) 800

Explanation: TPM = (12,000 × 2) / 30 = 24,000 / 30 = 800.

Question 57

Using the Cleaning Efficiency formula (Trash Extracted ÷ Total Trash Content in Raw Cotton × 100), if trash extracted = 15 g and total trash content = 60 g, the cleaning efficiency is:

A) 15%	C) 40%

B) 25%	D) 75%

Correct Answer: B) 25%

Explanation: (15 / 60) × 100 = 25%.

Question 58

The maximum delivery speed of a Draw Frame as per the PDF is:

A) 400 m/min	C) 1100 m/min

B) 800 m/min	D) 1500 m/min

Correct Answer: C) 1100 m/min

Explanation: Draw frame delivery speed is up to 1100 m/min (standard: 350–800 m/min).

Question 59

The Main Cylinder in carding has a diameter of approximately:

A) 250 mm	C) 1280–1300 mm

B) 500 mm	D) 2000 mm

Correct Answer: C) 1280–1300 mm

Explanation: The main cylinder is cast iron with a diameter of 1280–1300 mm (50 inches).

Question 60

The Licker-in speed for cotton is 800–1200 rpm, while for MMF it is:

A) 200 rpm	C) 1500 rpm

B) 600 rpm	D) 3000 rpm

Correct Answer: B) 600 rpm

Explanation: The PDF specifies licker-in speed of 800–1200 rpm for cotton and 600 rpm for MMF.

Question 61

The twist inserted by the Speed Frame flyer ranges from:

A) 5–10 TPM	C) 100–200 TPM

B) 30–65 TPM	D) 1000 TPM

Correct Answer: B) 30–65 TPM

Explanation: The flyer inserts a twist range of 30–65 TPM (turns per metre).

Question 62

The spindle speed range in Ring Spinning is:

A) 5,000–8,000 rpm	C) 50,000–75,000 rpm

B) 18,000–22,000 rpm	D) 100,000 rpm

Correct Answer: B) 18,000–22,000 rpm

Explanation: Ring spinning spindle speed is 18,000–22,000 rpm as per the Features section.

Question 63

The Ring Diameter in Ring Spinning ranges from:

A) 10–20 mm	C) 60–80 mm

B) 36–54 mm	D) 100–120 mm

Correct Answer: B) 36–54 mm

Explanation: Ring diameter in ring spinning ranges from 36–54 mm.

Question 64

The maximum Traveler Velocity in Ring Spinning is about:

A) 5–10 m/s	C) 100 m/s

B) 35–40 m/s	D) 200 m/s

Correct Answer: B) 35–40 m/s

Explanation: Traveler velocity reaches a maximum of 35–40 m/s.

Question 65

The Rotor Speed in Rotor Spinning can go up to:

A) 10,000 rpm	C) 150,000 rpm

B) 50,000 rpm	D) 500,000 rpm

Correct Answer: C) 150,000 rpm

Explanation: Under Features, rotor speed is stated as up to 150,000 rpm.

Question 66

The draft range in Air-Jet Spinning (MJS) is:

A) 5–22	C) 100–200

B) 60–70	D) 500–1000

Correct Answer: C) 100–200

Explanation: In Air-Jet Spinning, the drafted fibre strand undergoes a draft range of 100–200.

Question 67

The minimum number of fibres required in the cross-section for Air-Jet Spinning is:

A) 20	C) 80

B) 50	D) 150

Correct Answer: C) 80

Explanation: Air-Jet Spinning requires a minimum of 80 fibres in cross-section.

Question 68

Compact Spinning reduces yarn hairiness by up to:

A) 25%	C) 85%

B) 50%	D) 100%

Correct Answer: C) 85%

Explanation: Compact spinning reduces yarn hairiness by up to 85%.

Question 69

Compact Spinning increases yarn strength and elongation by up to:

A) 5%	C) 20%

B) 10%	D) 50%

Correct Answer: C) 20%

Explanation: Compact spinning increases yarn strength and elongation by up to 20% and allows about 10% twist reduction.

Question 70

The standard bundle weight in Post-Spinning Bundling is:

A) 1 kg or 2 kg	C) 10 kg or 12 kg

B) 4.5 kg or 5 kg	D) 20 kg

Correct Answer: B) 4.5 kg or 5 kg

Explanation: Standard bundle weight after reeling is 4.5 kg or 5 kg.

Question 71

The Sliver Lap Machine can produce up to:

A) 100 kg/hr	C) 480 kg/hr

B) 300 kg/hr	D) 800 kg/hr

Correct Answer: C) 480 kg/hr

Explanation: The Sliver Lap Machine production is up to 480 kg/hr.

Question 72

The Super Lap / Lap Former can produce up to:

A) 100 kg/hr	C) 480 kg/hr

B) 300 kg/hr	D) 800 kg/hr

Correct Answer: B) 300 kg/hr

Explanation: The Super Lap/Lap Former has a production rate up to 300 kg/hr.

Question 73

The maximum production rate of a Draw Frame per delivery is:

A) 100 kg/hr	C) 400 kg/hr

B) 250 kg/hr	D) 600 kg/hr

Correct Answer: C) 400 kg/hr

Explanation: Draw frame production rate is up to 400 kg/hr per delivery.

Question 74

The Scutcher Lap Weight tolerance is:

A) ±14 g to ±25 g per 36.6 kg lap	C) ±500 g to ±700 g per 36.6 kg lap

B) ±114 g to ±225 g per 36.6 kg lap	D) No tolerance is allowed

Correct Answer: B) ±114 g to ±225 g per 36.6 kg lap

Explanation: This exact tolerance value is given under Scutcher Lap Tolerances.

Question 75

The Equivalent Twist of a TFO Twister ranges from:

A) 2,000–4,000 TPM	C) 24,000–28,000 TPM

B) 12,000–14,000 TPM	D) 50,000 TPM

Correct Answer: C) 24,000–28,000 TPM

Explanation: TFO Equivalent Twist is 24,000–28,000 TPM (Spindle Speed is 12,000–14,000 rpm, a separate value).

SECTION D — STATEMENT-BASED (76–100)

Question 76

Statement I: Roller ginning uses a push-pull-hit action to separate fibres from seeds.

Statement II: Saw ginning uses rotating circular saws to pull fibres away from seeds at high speed.

A) Both Statement I and Statement II are correct	C) Statement I is correct, but Statement II is incorrect

B) Both Statement I and Statement II are incorrect	D) Statement I is incorrect, but Statement II is correct

Correct Answer: A) Both Statement I and Statement II are correct

Question 77

Statement I: Roller ginning preserves maximum natural fibre length.

Statement II: Saw ginning generates more neps and increases short fibre content.

A) Both Statement I and Statement II are correct	C) Statement I is correct, but Statement II is incorrect

B) Both Statement I and Statement II are incorrect	D) Statement I is incorrect, but Statement II is correct

Correct Answer: A) Both Statement I and Statement II are correct

Explanation: Both are listed correctly under advantages of roller ginning and disadvantages of saw ginning respectively.

Question 78

Consider the following statements about ginning:

Roller gins produce around 250 kg/hour.

Saw gins produce 500 to 2000 kg per hour.

Roller ginning is faster than saw ginning.

Saw ginning requires a qualified, experienced technician.

Which of the above statements are correct?

A) 1, 2 and 4 only	C) 2 and 3 only

B) 1 and 3 only	D) 1, 2, 3 and 4

Correct Answer: A) 1, 2 and 4 only

Explanation: Statement 3 is incorrect — the PDF states roller ginning is slower compared to saw ginning.

Question 79

Statement I: Mixing combines materials whose physical properties are only partially known. Statement

II: Blending combines materials in precise, measured, and controlled proportions.

A) Both Statement I and Statement II are correct	C) Statement I is correct, but Statement II is incorrect

B) Both Statement I and Statement II are incorrect	D) Statement I is incorrect, but Statement II is correct

Correct Answer: A) Both Statement I and Statement II are correct

Explanation: Both statements correctly reflect the PDF's definitions of mixing and blending.

Question 80

Statement I: The Uni-Mixer supplies material pneumatically into 8 vertical trunks/chutes.

Statement II: The Step Cleaner uses six beaters inclined at 45° with elliptical spikes.

A) Both Statement I and Statement II are correct	C) Statement I is correct, but Statement II is incorrect

B) Both Statement I and Statement II are incorrect	D) Statement I is incorrect, but Statement II is correct

Correct Answer: A) Both Statement I and Statement II are correct

Explanation: Both machine descriptions match the Step-by-Step Working Process section of the PDF.

Question 81

Consider the following statements about opening and cleaning machines:

Mono Cylinder Cleaner has 8 striker blades.

Mono Cylinder Cleaner runs at 700 rpm.

Step Cleaner effective speed is around 550 rpm.

Multi-Mixer has 2 to 4 vertical chambers.

Which of the above statements are correct?

A) 1, 2 and 3 only	C) 1 and 4 only

B) 2, 3 and 4 only	D) 1, 2, 3 and 4

Correct Answer: A) 1, 2 and 3 only

Explanation: Statement 4 is incorrect — the Multi-Mixer has 6 to 8 vertical chambers, not 2 to 4.

Question 82

Statement I: The Blowroom uses air currents for pneumatic material transport. Statement II: The Blowroom is positioned after the carding machine in the spinning sequence.

A) Both Statement I and Statement II are correct	C) Statement I is correct, but Statement II is incorrect

B) Both Statement I and Statement II are incorrect	D) Statement I is incorrect, but Statement II is correct

Correct Answer: C) Statement I is correct, but Statement II is incorrect

Explanation: The Blowroom is the first section, positioned before carding, not after it.

Question 83

Statement I: In the Scutcher, the beater rotates at 750–1100 rpm.

Statement II: The condensed fibre sheet is compressed by heavy calendar rollers into a compact lap sheet.

A) Both Statement I and Statement II are correct	C) Statement I is correct, but Statement II is incorrect

B) Both Statement I and Statement II are incorrect	D) Statement I is incorrect, but Statement II is correct

Correct Answer: A) Both Statement I and Statement II are correct

Explanation: Both facts are stated in the Scutcher/Lap Former working process.

Question 84

Statement I: Chute Feed eliminates lap formation and lap faults such as lap licking and lap splitting.

Statement II: Chute Feed makes it easier to control medium- and long-term sliver weight variation compared to weighing individual laps.

A) Both Statement I and Statement II are correct	C) Statement I is correct, but Statement II is incorrect

B) Both Statement I and Statement II are incorrect	D) Statement I is incorrect, but Statement II is correct

Correct Answer: C) Statement I is correct, but Statement II is incorrect

Explanation: Chute Feed actually makes it more difficult to control sliver weight variation compared to weighing individual laps — this is listed as a limitation.

Question 85

Statement I: In carding action, teeth of the two opposing surfaces face in opposite directions.

Statement II: In stripping action, teeth of both clothing surfaces face in the same direction.

A) Both Statement I and Statement II are correct	C) Statement I is correct, but Statement II is incorrect

B) Both Statement I and Statement II are incorrect	D) Statement I is incorrect, but Statement II is correct

Correct Answer: A) Both Statement I and Statement II are correct

Explanation: These are the exact tooth-direction principles for carding action and stripping action given in the PDF.

Question 86

Consider the following statements about carding:

The main cylinder rotates at about 600 rpm (up to 600–900 rpm on modern cards).

The licker-in diameter is 250 mm.

The cylinder to doffer setting is about 5 mm.

About 50% of fibres form trailing hooks after carding.

Which of the above statements are correct?

A) 1, 2 and 4 only	C) 2 and 3 only

B) 1, 2, 3 and 4	D) 1 and 3 only

Correct Answer: A) 1, 2 and 4 only

Explanation: Statement 3 is incorrect — the cylinder to doffer setting is about 0.1 mm, not 5 mm.

Question 87

Statement I: Drawing involves doubling and roller drafting of slivers.

Statement II: Roller drafting is the process of attenuating material count using pairs of rollers.

A) Both Statement I and Statement II are correct	C) Statement I is correct, but Statement II is incorrect

B) Both Statement I and Statement II are incorrect	D) Statement I is incorrect, but Statement II is correct

Correct Answer: A) Both Statement I and Statement II are correct

Explanation: Both are exact definitions given under the Draw Frame section.

Question 88

Statement I: The draw frame doubling formula reduces CV by averaging thick and thin places over several strands.

Statement II: A second drawframe passage always yields significant further improvement in evenness beyond the first passage.

A) Both Statement I and Statement II are correct	C) Statement I is correct, but Statement II is incorrect

B) Both Statement I and Statement II are incorrect	D) Statement I is incorrect, but Statement II is correct

Correct Answer: C) Statement I is correct, but Statement II is incorrect

Explanation: The PDF states a 2nd drawframe passage yields minimal further evenness improvement, while a 3rd passage can cause quality deterioration.

Question 89

Statement I: Carded slivers contain more than 50% trailing hooks.

Statement II: Rectilinear combers straighten and remove only leading hooks.

A) Both Statement I and Statement II are correct	C) Statement I is correct, but Statement II is incorrect

B) Both Statement I and Statement II are incorrect	D) Statement I is incorrect, but Statement II is correct

Correct Answer: A) Both Statement I and Statement II are correct

Explanation: Both statements are part of the Hook Reversal Principle in Comber Preparatory.

Question 90

Consider the following statements about Comber Preparatory:

The Conventional Method uses a Sliver Lap Machine followed by a Ribbon Lap Machine.

The Modern Method uses a Draw Frame followed by a Lap Former/Super Lap.

The Super Lap can double up to 60 slivers.

The Ribbon Lap Machine has a draft range of 1.31–3.03.

Which of the above statements are correct?

A) 1, 2 and 3 only	C) 2 and 4 only

B) 1, 2, 3 and 4	D) 1 and 4 only

Correct Answer: A) 1, 2 and 3 only

Explanation: Statement 4 is incorrect — a draft range of 1.31–3.03 belongs to the Sliver Lap Machine; the Ribbon Lap Machine's draft range is 3.48–8.04.

Question 91

Statement I: The Comber removes a precisely pre-determined quantity of short fibres known as noil.

Statement II: Backward/Counter Feed combers are used mainly for short staple fibres.

A) Both Statement I and Statement II are correct	C) Statement I is correct, but Statement II is incorrect

B) Both Statement I and Statement II are incorrect	D) Statement I is incorrect, but Statement II is correct

Correct Answer: C) Statement I is correct, but Statement II is incorrect

Explanation: Backward/Counter Feed is used for long staple fibres, not short staple; Forward/Concurrent Feed is used for short and medium staple fibres.

Question 92

Statement I: In Forward/Concurrent Feed combing, noil ranges from 6–18%. Statement II: In Backward/Counter Feed combing, noil ranges from 14–25%.

A) Both Statement I and Statement II are correct	C) Statement I is correct, but Statement II is incorrect

B) Both Statement I and Statement II are incorrect	D) Statement I is incorrect, but Statement II is correct

Correct Answer: A) Both Statement I and Statement II are correct

Explanation: Both noil ranges match the PDF's classification by feed mechanism.

Question 93

Statement I: The Speed Frame inserts a slight protective twist of 30–65 TPM. Statement II: In the Flyer Leading System, flyer speed is greater than bobbin speed and bobbin speed increases as the bobbin fills.

A) Both Statement I and Statement II are correct	C) Statement I is correct, but Statement II is incorrect

B) Both Statement I and Statement II are incorrect	D) Statement I is incorrect, but Statement II is correct

Correct Answer: A) Both Statement I and Statement II are correct

Explanation: Both facts are given in the Speed Frame Objectives and Types sections.

Question 94

Consider the following statements about Ring Spinning:

Twist is inserted by a traveler moving around a ring.

The spindle speed ranges from 18,000–22,000 rpm.

The traveler rotates faster than the spindle, causing winding.

Total draft can go up to 70–75.

Which of the above statements are correct?

A) 1, 2 and 4 only	C) 1 and 3 only

B) 1, 2, 3 and 4	D) 2 and 3 only

Correct Answer: A) 1, 2 and 4 only

Explanation: Statement 3 is incorrect — winding occurs because the traveler rotates slightly slower than the spindle due to friction, not faster.

Question 95

Statement I: In TFO, the first twist is inserted between the top entry guide and spindle rotor exit.

Statement II: In TFO, the second twist is inserted in the ballooning zone between the reserve disc exit and stationary pigtail guide.

A) Both Statement I and Statement II are correct	C) Statement I is correct, but Statement II is incorrect

B) Both Statement I and Statement II are incorrect	D) Statement I is incorrect, but Statement II is correct

Correct Answer: A) Both Statement I and Statement II are correct

Explanation: These statements exactly match the Twist Formation description of the TFO in the PDF.

Question 96

Statement I: TFO eliminates the need for ring and traveler.

Statement II: TFO requires rewinding after twisting, unlike ring doubling.

A) Both Statement I and Statement II are correct	C) Statement I is correct, but Statement II is incorrect

B) Both Statement I and Statement II are incorrect	D) Statement I is incorrect, but Statement II is correct

Correct Answer: C) Statement I is correct, but Statement II is incorrect

Explanation: TFO actually eliminates rewinding — this is listed as an advantage of the TFO system.

Question 97

Consider the following statements about Non-Conventional Spinning:

Rotor spinning eliminates the Speed Frame and Winding processes.

Air-Jet Spinning yarn has only 50–60% of the strength of ring yarn for 100% cotton.

Air Vortex Spinning yarn has a lower percentage of wrapper fibres than Air-Jet yarn.

Compact Spinning almost eliminates the spinning triangle.

Which of the above statements are correct?

A) 1, 2 and 4 only	C) 1 and 4 only

B) 1, 2, 3 and 4	D) 2 and 3 only

Correct Answer: A) 1, 2 and 4 only

Explanation: Statement 3 is incorrect — Air Vortex Spinning yarn has a HIGHER percentage of wrapper fibres than Air-Jet yarn, as per the Features section.

Question 98

Statement I: Compact spinning allows about a 10% twist reduction.

Statement II: Compact spinning increases traveler wear due to metallic contact.

A) Both Statement I and Statement II are correct	C) Statement I is correct, but Statement II is incorrect

B) Both Statement I and Statement II are incorrect	D) Statement I is incorrect, but Statement II is correct

Correct Answer: A) Both Statement I and Statement II are correct

Explanation: Both are listed respectively under Advantages and Disadvantages of Compact Spinning.

Question 99

Statement I: Post-spinning processes begin with yarn conditioning to stabilize twist and prevent snarling.

Statement II: Reeling converts yarn from spinning cops into skeins (hanks).

A) Both Statement I and Statement II are correct	C) Statement I is correct, but Statement II is incorrect

B) Both Statement I and Statement II are incorrect	D) Statement I is incorrect, but Statement II is correct

Correct Answer: A) Both Statement I and Statement II are correct

Explanation: Both statements correctly follow the Post-Spinning Process Sequence (Step 1 and Step 2).

Question 100

Consider the following statements about Post-Spinning output packages:

Cones

Cheeses

Hanks

Laps

Which of the above are listed as output packages in Post-Spinning Processes?

A) 1, 2 and 3 only	C) 2 and 4 only

B) 1, 2, 3 and 4	D) 1 and 4 only

Correct Answer: A) 1, 2 and 3 only

Explanation: The PDF lists Output Packages as Cones, Cheeses, Hanks, and Bales — "Laps" is not part of the post-spinning output list (laps belong to the blowroom/comber preparatory stages).